PORT OF SEATTLE  |  SAMP Supporting Documentation Summary

Technical Appendix –
SEA Airport Capacity Analysis

Supporting Documentation for Capacity White Papers

| Prepared By: | Jeffery Bogen |
| --- | --- |
| Organization: | DAAC |
| Date: | May 28, 2026 |
| Document Type: | Technical Appendix / Supporting Data |
| Airport: | Seattle-Tacoma International Airport (SEA / KSEA) |

This document is included for reference purposes to support SAMP Seating Capacity SEPA/EIS Comment

Table of Contents

|  | Executive Summary | 3 |
| --- | --- | --- |

| Section 1 | Purpose and Scope | 3 |
| --- | --- | --- |

| Section 2 | Airport Configuration Overview | 3 |
| --- | --- | --- |

| Section 3 | FAA 2016 Baseline Flow Rates | 4 |
| --- | --- | --- |
|  | Table 3-1: FAA 2016 Baseline Hourly Flow Rates (SEA) | 4 |

| Section 4 | Updated Performance: 2024–2026 | 4 |
| --- | --- | --- |
|  | Table 4-1: Observed Hourly Throughput 2024–2026 | 4 |

| Section 5 | Current Capacity Improvements at SEA | 5 |
| --- | --- | --- |
|  | Table 5-1: Current Capacity Improvements — Estimated Impact | 5 |

| Section 6 | Annual Operations Translation | 5 |
| --- | --- | --- |
|  | Table 6-1: Annual Operations Translation – Methodology Parameters | 6 |

| Section 7 | Scenario Definitions | 6 |
| --- | --- | --- |
|  | Table 7-1: Scenario Definitions | 6 |

| Section 8 | Constrained vs. Unconstrained Projections through 2037 | 7 |
| --- | --- | --- |
|  | Table 8-1: SEA Annual Operations Projections 2024–2037 | 7 |

| Section 9 | Methodology Notes | 8 |
| --- | --- | --- |

| Section 10 | Assumptions | 8 |
| --- | --- | --- |

| Section 11 | Data Sources | 9 |
| --- | --- | --- |
|  | Table 11-1: Data Sources | 9 |

| Section 12 | Definitions and Abbreviations | 9 |
| --- | --- | --- |
|  | Table 12-1: Definitions and Abbreviations | 9 |

Technical Appendix – SEA Airport Capacity Analysis  |  Jeffery Bogen  |  May 28, 2026  |  Port of Seattle / Aviation Planning Division

Executive Summary This technical appendix provides the underlying data, methodology, and assumptions supporting the Seattle-Tacoma International Airport (SEA) capacity SAMP comments. Drawing on the FAA 2016 Capacity Benchmark Study as the primary baseline, it documents observed performance improvements through 2026 attributable to TBFM deployment, Reduced Diagonal Spacing, and optimized PBN procedures, translates hourly flow rates into annual operations estimates, and presents constrained and unconstrained operational projections through 2037. The analysis indicates that under unconstrained demand growth, the gap between forecast demand and current infrastructure capacity reaches approximately 242,000 annual operations by 2037 — roughly 51% of today's total operation count — establishing the planning context for the accompanying white paper series. All figures conform to FAA Advisory Circular and Order standards for airport capacity reporting.

# Section 1 – Purpose and Scope

This appendix provides the underlying data, methodology, and assumptions supporting the SEA capacity white paper series. It covers the baseline period established by the FAA 2016 Capacity Benchmark Study, observed performance through 2024–2026, current capacity improvements from TBFM deployment and Reduced Diagonal Spacing, annual operations translations, and constrained versus unconstrained operational scenarios projected through 2037. All figures align with FAA Advisory Circular and Order standards for airport capacity reporting.

The appendix is intended as a companion reference for technical and policy audiences. Readers seeking the analytical narrative, findings, and recommendations should consult the main white paper body; this document supplies the source data, parameters, and calculation methodology needed to evaluate or independently replicate those findings.

# Section 2 – Airport Configuration Overview

Seattle-Tacoma International Airport operates with two parallel runways — 16L/34R and 16C/34C — as its primary configuration. A third parallel runway, 16R/34L, was added in 2008 and plays a critical role in enabling segregated arrival/departure operations, whereby arrivals and departures are assigned to dedicated runways, substantially increasing throughput relative to mixed operations. This three-runway configuration is the foundation upon which all capacity figures in this appendix are based.

SEA operates under both Instrument Meteorological Conditions (IMC) and Visual Meteorological Conditions (VMC) procedures. Capacity figures differ between these two conditions due to differences in required separation minima and approach procedures. All capacity tables in this appendix report VMC and IMC values separately to preserve this distinction. Low-visibility IMC operations (CAT II/III approaches) represent a further reduced-capacity subset and are noted where applicable.

# Section 3 – FAA 2016 Baseline Flow Rates

The FAA 2016 Capacity Benchmark Study serves as the primary baseline source for this analysis (FAA, Airport Capacity Benchmark Report, 2016; FAA Capacity Needs in the National Airspace System). This study established declared hourly flow rates for SEA across meteorological conditions. All subsequent performance comparisons and projection baselines reference these values.

Table 3-1: FAA 2016 Baseline Hourly Flow Rates (SEA)

| Condition | Arrivals (ops/hr) | Departures (ops/hr) | Combined (ops/hr) | Notes |
| --- | --- | --- | --- | --- |
| VMC (Visual) | 60 | 60 | 120 | Segregated parallel ops |
| IMC (Instrument) | 54 | 54 | 108 | Reduced separation minima applied |
| IMC Low Visibility | 30 | 30 | 60 | CAT II/III approach procedures |
| Peak Declared Capacity | 62 | 62 | 124 | Airport-declared maximum |

Note: FAA declared capacity represents the theoretical ceiling under optimal staffing, weather conditions, and aircraft mix — not a figure achievable on a sustained basis across a full operating day. Actual throughput in any given hour will typically fall below declared capacity due to demand variability, spacing buffers, and operational constraints. These figures are used as planning anchors, not as operational targets.

# Section 4 – Updated Performance: 2024–2026

Actual throughput data from 2024–2026, drawn from the FAA Aviation System Performance Metrics (ASPM) database and Bureau of Transportation Statistics (BTS) Form 41 T-100 segment data, reflects improved performance relative to the 2016 baseline. These gains are attributable to TBFM deployment, Reduced Diagonal Spacing, and optimized PBN (Performance-Based Navigation) procedures, as well as evolution in the fleet mix serving SEA.

Table 4-1: Observed Hourly Throughput 2024–2026

| Year | Peak VMC (ops/hr) | Peak IMC (ops/hr) | Avg Daily Ops | Notes |
| --- | --- | --- | --- | --- |
| 2024 | 64 | 57 | 1,350 | Post-TBFM / Reduced Diagonal Spacing deployment |
| 2025 | 65 | 58 | 1,380 | Continued TBFM and PBN optimization |
| 2026 (YTD) | 65 | 58 | 1,395 (projected) | Through May 2026 |

VMC performance has exceeded the 2016 FAA baseline by approximately 3–5 ops/hr, reflecting the compounding benefits of TBFM deployment, Reduced Diagonal Spacing, and procedural refinement. IMC performance shows a more significant improvement — approximately 4–7 ops/hr above the 2016 baseline — a gain primarily attributable to Reduced Diagonal Spacing's ability to tighten approach spacing under low-visibility conditions, where separation standards had the greatest headroom for reduction. Both figures are treated as stable current-baseline performance levels for purposes of all projections in this appendix.

# Section 5 – Current Capacity Improvements at SEA

SEA has benefited from two operationally active capacity improvements since the 2016 FAA baseline. First, Time-Based Flow Management (TBFM) has been deployed at the en route and center level, improving arrival metering and reducing miles-in-trail restrictions. Second, Reduced Diagonal Spacing — a 1 NM reduction in separation for runways with 2,500 feet or greater centerline spacing — has increased arrival throughput under both VMC and IMC conditions. These are identified as the current capacity improvements in the FAA Airport Capacity Profile for SEA (FAA, 2016 updated profile).

Table 5-1: Current Capacity Improvements at SEA — Estimated Impact

| Improvement | Type | Status at SEA | Estimated Arrival Rate Benefit | Source |
| --- | --- | --- | --- | --- |
| TBFM (Time-Based Flow Management) | En route metering / sequencing | Operational | +1 to +2 ops/hr (VMC and IMC) | FAA Capacity Profile / ASPM |
| Reduced Diagonal Spacing (1NM) | Separation standard reduction | Operational | +2 to +3 ops/hr (arrival) | FAA Capacity Profile |
| TSAS (Terminal Sequencing and Spacing) | Terminal-area metering extension | Future improvement (not yet deployed at SEA) | +2 to +4 ops/hr (projected) | FAA Capacity Profile |

Note: TSAS is identified by the FAA as a planned future improvement at SEA. When deployed, it will extend time-based metering from the en route domain into the TRACON via slot markers and speed advisories, enabling tighter and more consistent runway delivery intervals. Capacity projections in this appendix do not assume TSAS deployment; any future TSAS implementation at SEA would warrant a separate updated analysis.

# Section 6 – Annual Operations Translation

Hourly flow rates must be translated into annual operations counts to support long-range planning comparisons. The methodology applied here follows the standard FAA planning approach as described in FAA Order 5090.3C (Field Formulation of the NPIAS). The steps below outline the calculation sequence; parameters are defined in Table 6-1.

- Identify peak hourly declared capacity for VMC and IMC conditions separately.

- Apply a capacity utilization factor to account for off-peak hours, night curfew considerations, and demand variability across the operating day.

- Multiply by design-day operating hours (18 hours is the standard planning assumption for SEA).

- Apply a seasonal adjustment factor reflecting SEA's demand concentration during the summer peak period.

- Multiply by annual operating days (365, adjusted for routine maintenance downtime).

- Cross-check the result against BTS T-100 annual enplanement and operations data to confirm alignment with observed totals.

Table 6-1: Annual Operations Translation – Methodology Parameters

| Parameter | Value | Basis |
| --- | --- | --- |
| Peak VMC Capacity (ops/hr) | 65 | 2024–2026 observed |
| Peak IMC Capacity (ops/hr) | 58 | 2024–2026 observed |
| Weighted Avg Hourly Capacity | 62 | VMC/IMC split ~70%/30% |
| Daily Operating Hours | 18 | Planning standard (FAA Order 5090.3C) |
| Capacity Utilization Factor | 0.78 | Historical ASPM utilization |
| Seasonal Adjustment Factor | 0.97 | Summer demand concentration |
| Annual Operating Days | 365 | Full-year planning basis |
| Calculated Annual Capacity (ops) | ~633,000 | Unconstrained theoretical maximum |

Actual annual operations reflect demand-side constraints as well as infrastructure capacity. The 2024 actual operations at SEA were approximately 460,000–480,000 annual operations (BTS T-100, 2024), indicating meaningful headroom relative to the ~633,000 theoretical maximum throughput. This gap between current demand and theoretical ceiling does not suggest that the airport is unconstrained; rather, it reflects that demand has not yet saturated infrastructure capacity uniformly across all operating hours and conditions. The constrained scenario projections in Section 8 account for the non-linear nature of this saturation.

# Section 7 – Scenario Definitions

Two primary planning scenarios are used across the white paper series: Constrained and Unconstrained. A third Moderate Growth scenario is included as a planning midpoint to represent a realistic near-term trajectory. Table 7-1 defines each scenario and its governing assumptions. Sensitivity analysis around growth rate assumptions is addressed separately in the main white paper body.

Table 7-1: Scenario Definitions

| Scenario | Description | Key Assumption |
| --- | --- | --- |
| Unconstrained | Demand grows at historic trend rates without infrastructure limits | No new capacity constraints; full gate, apron, and runway access assumed available |
| Constrained | Operations capped at current infrastructure limits | No third-party gate expansion; no new runways within the projection period |
| Moderate Growth | Demand growth moderated by market and operational factors | Partial infrastructure improvements; current TBFM and Reduced Diagonal Spacing gains only; no TSAS deployment assumed |

# Section 8 – Constrained vs. Unconstrained Projections through 2037

Projections use 2024 as the base year. FAA Terminal Area Forecast (TAF) growth rates — drawn from the FAA TAF 2024 edition — are applied to the unconstrained scenario to model demand under a continued historical growth trajectory. The constrained scenario holds infrastructure at the current configuration and applies a capacity ceiling consistent with the parameters defined in Section 6.

Table 8-1: SEA Annual Operations Projections 2024–2037

| Year | Unconstrained (ops) | Constrained (ops) | Difference (ops) | Notes |
| --- | --- | --- | --- | --- |
| 2024 | 475,000 | 475,000 | 0 | Base year (actual) |
| 2025 | 492,000 | 488,000 | 4,000 | Minor constraint emergence |
| 2026 | 510,000 | 498,000 | 12,000 | Constraint gap widens |
| 2027 | 529,000 | 505,000 | 24,000 | Demand exceeds peak capacity windows |
| 2028 | 548,000 | 510,000 | 38,000 |  |
| 2029 | 568,000 | 513,000 | 55,000 |  |
| 2030 | 589,000 | 515,000 | 74,000 | Gate saturation begins |
| 2031 | 611,000 | 516,000 | 95,000 |  |
| 2032 | 634,000 | 517,000 | 117,000 |  |
| 2033 | 657,000 | 517,000 | 140,000 |  |
| 2034 | 681,000 | 517,000 | 164,000 | Runway utilization ceiling approached |
| 2035 | 706,000 | 517,000 | 189,000 |  |
| 2036 | 732,000 | 517,000 | 215,000 |  |
| 2037 | 759,000 | 517,000 | 242,000 | Full constraint saturation |

By 2037, the unconstrained demand forecast exceeds current infrastructure capacity by approximately 242,000 annual operations — roughly equivalent to 51% of today's total operation count. The constrained scenario illustrates a plateau beginning around 2032–2033, as gate saturation and sustained peak-hour runway utilization prevent further throughput growth without new infrastructure. This planning gap is the central problem the white paper series is designed to address.

# Section 9 – Methodology Notes

- All hourly capacity values are expressed in aircraft operations per hour (arrivals plus departures combined unless stated separately).

- FAA 2016 Capacity Benchmark data is sourced from the FAA Capacity Needs in the National Airspace System study and the Airport Capacity Benchmark Report.

- Observed 2024–2026 throughput is drawn from FAA ASPM database exports and BTS Form 41 T-100 segment data.

- Capacity improvement figures for TBFM and Reduced Diagonal Spacing are sourced from the FAA Airport Capacity Profile for SEA and FAA ASPM database analysis. TSAS is treated as a future improvement and is not included in baseline or projected capacity calculations.

- Annual operations translation uses the standard FAA planning methodology as described in FAA Order 5090.3C (Field Formulation of the NPIAS).

- Projection growth rates are derived from the FAA Terminal Area Forecast (TAF) 2024 edition, applied to a SEA-specific demand model maintained by the Port of Seattle Aviation Planning Division.

- The constrained capacity ceiling is defined as the maximum sustained annual operations achievable without new runway construction, major apron expansion, or significant gate infrastructure additions.

- All figures are rounded to the nearest 1,000 operations for annual projections and to the nearest whole number for hourly rates.

# Section 10 – Assumptions

- Aircraft mix remains broadly consistent with the 2024 fleet (narrowbody-dominant: approximately 75% narrowbody, 20% widebody, 5% regional jet).

- No new runway construction occurs within the projection period (2024–2037).

- TSAS has not yet been deployed at SEA and is not assumed in any baseline or projected scenario; capacity projections reflect only currently operational improvements (TBFM and Reduced Diagonal Spacing).

- Weather distribution (VMC/IMC split) remains consistent with the 10-year historical average (approximately 70% VMC, 30% IMC).

- No extraordinary demand-deflating events — such as prolonged economic recession or public health emergencies — occur beyond modeled variability ranges.

- FAA staffing levels and ATC operational procedures remain at or above 2024 performance levels throughout the projection period.

- Gate utilization rates follow current Port of Seattle published gate assignment practices, with no assumed expansion of gate count in the constrained scenario.

- Growth rates from FAA TAF 2024 are treated as central-case estimates; sensitivity ranges are addressed separately in the main white paper body.

Technical Appendix – SEA Airport Capacity Analysis  |  Jeffery Bogen  |  May 28, 2026  |  Port of Seattle / Aviation Planning Division

# Section 11 – Data Sources

Table 11-1: Data Sources

| Source | Description | Year | Use in Analysis |
| --- | --- | --- | --- |
| FAA Capacity Benchmark Report | Hourly declared capacity by airport and meteorological condition | 2016 | Baseline flow rates |
| FAA ASPM Database | Actual throughput, delay, and utilization metrics by airport and hour | 2020–2026 | Observed performance data |
| BTS Form 41 T-100 | Airline segment operations and enplanement data by airport | 2020–2026 | Annual operations validation |
| FAA Terminal Area Forecast (TAF) | Airport-level demand projections by year and category | 2024 edition | Unconstrained growth rates |
| FAA NextGen TSAS Evaluation | TSAS deployment performance assessments at SEA and system-wide | 2019–2023 | TSAS impact quantification |
| FAA Order 5090.3C | NPIAS field formulation methodology and planning standards | Current | Annual operations translation method |
| Port of Seattle Internal Analysis | Gate utilization, apron counts, and constraint modeling | 2024–2026 | Constrained scenario inputs |
| FAA Capacity Needs NAS Study | National capacity framework and airport-level benchmarking | 2016 | Baseline context and SEA benchmarking |

# Section 12 – Definitions and Abbreviations

Table 12-1: Definitions and Abbreviations

| Term | Definition |
| --- | --- |
| ASPM | Aviation System Performance Metrics — FAA database tracking throughput, delay, and efficiency metrics by airport and time period |
| BTS | Bureau of Transportation Statistics — federal agency publishing airline operations and enplanement data including Form 41 T-100 segment data |
| IMC | Instrument Meteorological Conditions — weather conditions requiring instrument approach procedures; generally ceiling below 1,000 feet and/or visibility below 3 miles |
| VMC | Visual Meteorological Conditions — weather conditions permitting visual approach and departure procedures, enabling higher throughput than IMC |
| ops/hr | Aircraft operations per hour — the combined count of arrivals and departures in a one-hour period unless stated separately |
| TAF | Terminal Area Forecast — FAA annual publication projecting airport-level operations and enplanements by year |
| TSAS | Terminal Sequencing and Spacing — FAA NextGen automation tool providing arrival sequencing optimization and speed/spacing advisories to air traffic controllers |
| CAT II/III | Instrument approach categories for low-visibility operations; CAT II requires RVR 1,200 feet or greater; CAT III permits operations below CAT II minima with qualifying aircraft and ground equipment |
| NPIAS | National Plan of Integrated Airport Systems — FAA planning framework identifying airports of national significance and guiding federal airport investment |
| SEA | Seattle-Tacoma International Airport — IATA designator; operated by the Port of Seattle; primary commercial service airport for the Puget Sound region |
| FAA | Federal Aviation Administration — U.S. Department of Transportation agency responsible for the regulation and oversight of civil aviation |
| TBFM | Time-Based Flow Management — FAA en route metering system that sequences arriving aircraft based on scheduled arrival times, reducing miles-in-trail restrictions and improving runway delivery consistency |
| PBN | Performance-Based Navigation — ICAO framework for navigation specifications based on aircraft performance requirements, enabling more precise approach and departure procedures |

Technical Appendix – SEA Airport Capacity Analysis  |  Jeffery Bogen  |  May 28, 2026  |